上海地区深基坑周围地面沉降特点及其预测

杨敏1,2，卢俊义 1,2

(1. 同济大学 岩土及地下工程教育部重点实验室，上海 200092; 2. 同济大学 地下建筑与工程系，上海 200092;)

摘  要：通过研究上海地区32个深基坑工程的监测数据，得到以下地面沉降特点：大多数地面沉降曲线呈凹槽型，存在沉降槽偏心距；最大沉降量的变化范围在0.4～1.9之间，其平均值和最大侧向位移相等；所有基坑的支撑系统相对刚度都较大，支撑系统相对刚度对最大沉降量影响很小；抗隆起安全系数增大，基坑最大沉降量减小，同时绘制了基于抗隆起安全系数的最大沉降量预测曲线。最后，基于上海地区深基坑的沉降实测曲线特征，提出了考虑沉降槽偏心距的地面沉降预测方法，并结合四个工程实例进行了验证，结果表明，该方法预测精度较高，是一种实用的基坑周围地面沉降预测方法。

关键词：深基坑；地面沉降；监测数据；预测

中文分类号:              文献标示码:A        文章编号：0253-374x（2010）02-

Characteristics and Prediction of Ground Settlement around

Deep Excavation in Shanghai

YANG Min 1,2, LU Jun-Yi 1,2

(1.Key Laboratory of Geotechnical and Underground Engineering of Ministry of Education; Tongji University ,Shanghai, 200092，China

2.Dept. of Geotech. Eng., Tongji University ,Shanghai, 200092，China)

Abstract: Based on the field monitoring data of 32 deep excavations in Shanghai, the main characteristics of ground settlement have been summarized as follows. Firstly, most settlement profiles around deep excavations appear to be grooves with settling tanks. Secondly, the maximum ground settlement,, lies between 0.4 and 1.9 (for the maximum lateral deflection), and its mean value is equal to . Thirdly, system stiffness of all deep excavations is great and its influence on is small. Fourthly, the maximum ground settlement decreases with the increase of the factor of safety against base heave (FOS), and the FOS-based settlement predictive curve is proposed at the same time. Finally, according to the character of measured settlement profiles in Shanghai, the author proposed a predictive method taking into account effect of the settling tank’s eccentricity. With the verification of the proposed method in four deep excavation cases, the results show a high degree of accuracy, which is proved to be a practical method in ground settlement prediction around deep excavation.

Key words: deep excavation; ground settlement; monitoring data; prediction

深基坑工程不仅要保证基坑本身的安全与稳定，而且要有效控制基坑周围地层移动以保护周围环境[1]。工程实践表明，在上海软土地区，深基坑开挖会引起周围地层较大的变形，这种变形往往表现为地表监测点的沉降。因此，基坑周围的地面沉降最大值是上海基坑工程设计规程中一个重要的控制指标。如果能够对深基坑周围地面沉降有效预测，则能科学的指导深基坑的设计工作。

深基坑周围地面沉降主要有两种预测方法，一种是建立在统计基础上的经验统计法；一种为数值模拟法。对于数值模拟法，受小应变问题的复杂性和实验条件的限制，目前这方面的研究多集中在理论研究或单个基坑的性状分析方面，实用的预测方法还很少。而经验统计法以其简单实用性而受到广大岩土工作者的青睐，国外或港台地区比较有代表性的研究者有Peck[2]、Clough[3]、欧章煜[4]等；在上海地区，早期的研究者有刘建航[5]、侯学渊[1]、唐孟雄[6]等，取得了大量研究成果。笔者首先收集了上海地区近年来的基坑监测资料，研究了基坑周围地面沉降的沉降特点，最后，提出了考虑沉降槽偏心距的深基坑周围地面沉降预测方法。

1  基坑工程实例

笔者收集了上海地区32个成功基坑资料，如表1所示。32个基坑全部采用连续墙加内支撑的围护结构型式，基坑的基本信息包括开挖深度、连续墙厚度、连续墙深度、连续墙刚度和支撑的平均间距。监测结果主要统计了每个基坑连续墙的最大侧向位移和墙后土体的最大沉降量。

表1上海地区深基坑工程实测数据

Tab. 1 Data of deep excavation case histories in Shanghai

| 工程名称 | 开挖深度/m | 墙厚 /m | 墙体深度/m | 墙体刚度 /(kN·m2·m-1) | 平均支撑间距/m | 最大侧移 /mm | 最大沉降 /mm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1线黄陂路车站[7] | 14.2 | 0.80 | 30.0 | 1280000 | N/A | 64.5 | 76.7 |
| R1上体馆站[7] | 14.0 | 0.80 | 26.0 | 1280000 | 3.2 | 50.0 | 10.0 |
| 上海铁路南站南广场[8] | 13.4 | 0.80 | 25.4 | 1280000 | 4.4 | 26.0 | 43.6 |
| 人民广场地下变电站[9] | 23.2 | 1.20 | 32.8 | 4320000 | N/A | 19.7 | 16.0 |
| 上海地铁上体馆主变电站[10] | 15.9 | 0.65 | 23.8 | 686560 | 3.2 | 33.0 | 54.7 |
| 上海徐汇商城[11] | 13.4 | 0.80 | 26.0 | 1280000 | 3.9 | 100.0 | 60.0 |
| 上海南洋广场[12] | 15.7 | 1.00 | 30.5 | 2500000 | 4.7 | 45.5 | 36.8 |
| 上海市金融中心[13] | 16.9 | 1.00 | 32.0 | 2500000 | 5.0 | 101.9 | 72.4 |
| 亚龙投资总部大楼[14] | 13.5 | 0.80 | 24.8 | 1280000 | 4.0 | 18.9 | 18.0 |
| 外环隧道浦西基坑[15] | 30.4 | 1.20 | 44.0 | 4320000 | 4.7 | 123.0 | 125.9 |
| 明珠二期宜山路站[16] | 15.5 | 0.60 | 28.0 | 540000 | 2.8 | 38.0 | 14.0 |
| M8线黄兴路车站[17] | 14.7 | 0.60 | 26.5 | 540000 | 3.5 | 39.2 | 70.0 |
| R2线河南中路站标准段[18] | 15.2 | 0.80 | 30.4 | 1280000 | 2.5 | 97.8 | 101 |
| R1线人民广场站[19] | 13.5 | 0.80 | 24.4 | 1280000 | N/A | 72.5 | 110.0 |
| R1线漕宝路站[20] | 12.3 | 0.65 | 18.5 | 686560 | 4.1 | 57.4 | 66.0 |
| 同济大学图书馆[21] | 8.9 | 0.60 | 16.5 | 540000 | 2.2 | 27.0 | 16.0 |
| 胜康廖氏大厦[22] | 11.0 | 0.80 | 22.3 | 1280000 | 3.3 | 89.0 | 114.3 |
| 上海三角地广场[22] | 12.5 | 0.80 | 25.5 | 1280000 | 3.9 | 120.0 | 113.0 |
| R1线徐家汇车站[23] | 17.0 | 0.60 | 30.0 | 1280000 | 3.1 | 167.0 | 117.0 |
| R2线中央公园站[24] | 15.0 | 0.60 | 26.0 | 540000 | 3.4 | 50.0 | 36.4 |
| M8线延吉中路站标准段[25] | 14.4 | 0.80 | 27.0 | 1280000 | 3.3 | 18.0 | 30.0 |
| 恒隆广场主楼[26] | 18.2 | 1.00 | 33.0 | 2500000 | 3.8 | 100.0 | 100.0 |
| 上海置地广场[27] | 13.7 | 0.80 | 28.0 | 1280000 | 4.0 | 52.0 | 78.0 |
| 金茂大厦主楼[28] | 19.3 | 1.00 | 36.0 | 2500000 | 4.1 | 81.0 | 155.0 |
| 长峰大厦裙楼[29] | 18.0 | 0.80 | 31.0 | 1280000 | 3.5 | 53.7 | 34.9 |
| 延长路地铁车站[30] | 15.2 | 0.80 | 26.3 | 1280000 | 3.8 | 46.3 | 31.6 |
| 共和新路地铁车站基坑* | 15.2 | 0.80 | 26.5 | 1280000 | 3.1 | 17.1 | 31.6 |
| M8线周家渡站* | 14.9 | 0.80 | 27.0 | 1280000 | N/A | 16.8 | 25.0 |
| 崇明越江通道工程* | 25.0 | 1.00 | 35.0 | 2500000 | 4.2 | 28.0 | 46.7 |
| 西藏路电力隧道1号基坑* | 12.0 | 0.80 | 25.0 | 1280000 | N/A | 14.0 | 8.5 |
| 东安路地铁车站基坑* | 14.8 | 0.80 | 19.0 | 1280000 | 3.1 | 21.4 | 35.7 |
| 新世纪商厦基坑工程* | 10.7 | 0.60 | 22.0 | 540000 | N/A | 38.2 | 16.0 |

注：表中标*基坑为上海地质调查研究院内部研究资料;N/A表示该项资料缺失

为了叙述方便，本文中出现的符号意义如下图所示：

图1 深基坑各符号含义

Fig. 1 Definitions of deep excavation variable

2  基坑周围地面沉降特点分析

基坑工程周围的地面沉降问题非常复杂，不同基坑的沉降特征不会完全相同。通常，主要对基坑的沉降曲线特征及最大沉降量进行研究，解决了这两个问题，基坑周围的地面沉降特点也就迎刃而解。

2.1  基坑周围地面沉降曲线特征

根据表1中8个基坑的沉降监测资料，笔者绘制了单个基坑的地面沉降曲线图，如图2所示。从图2可以看出，上海市基坑周围地面沉降曲线型式以凹槽型为主，少量呈抛物型或三角型，符合文献[1]的统计结果。抛物型沉降曲线的最大地面沉降量位置在基坑壁处，即x0=0；而凹槽型沉降曲线的最大地面沉降量位置距离基坑壁有一段距离，即x0＞0。

图2 基坑周围地面沉降曲线

Fig. 2 Curves of ground settlement surrounding deep excavation

2.2  最大地面沉降量特点

前人研究表明，最大沉降量和一个或几个主要因素存在相关关系，如欧章煜等通过对台湾地区基坑进行研究，认为最大地面沉降量δvm和最大侧向位移δhm相关 [31]，而Clough等通过大量有限元计算和实测资料统计绘制了最大沉降量与围护结构系统刚度、基坑底部抗隆起安全系数的关系图[3]。因此，笔者就最大侧向位移、支撑系统相对刚度和抗隆起安全系数三个因素与地面最大沉降量的关系进行相关分析。

2.2.1 最大地面沉降量与最大侧向位移的关系

图3给出了基坑最大侧向位移与最大沉降量之间的关系。由图3可见，除了地铁R1线上体馆站基坑外，δvm的变化范围都在0.4δhm～1.9δhm之间，其平均值和δhm相等。欧章煜对台湾10个基坑的统计结果表明，δvm在0.5δhm～1δhm之间变化，平均值为0.7δhm [4]。和台湾地区基坑相比，δvm的最小值相差不大，而最大值要大的多。这可能与两地的地质条件差异、施工工艺差异及基坑统计的数量有关。

图3 最大侧向位移与的最大地面沉降关系

Fig.3 Relationship between normalized maximum ground settlement (/H) and normalized maximum lateral deflection (/H)

2.2.2 最大地面沉降量与支撑系统相对刚度的关系

支撑系统相对刚度采用Clough等的定义，表达式为：

S=EI/hγw	（1）

式中，EI为地连墙的水平单宽抗弯刚度，h为平均支撑间距，w为水的重度。由图4可以看出，32个基坑的支撑系统相对刚度都较大，而且支撑系统相对刚度对基坑最大沉降量的影响并不明显。因此，在上海地区，对于采用地下连续墙加内支撑围护结构型式的基坑，在考虑最大地面沉降时，可以忽略基坑支撑系统相对刚度的影响。

图4 支撑系统相对刚度与最大地面沉降的关系

Fig. 4 Relationship between system stiffness (Ks) and normalized maximum ground settlement (δvm/H)

2.2.3  地面最大沉降与坑底抗隆起安全系数的关系

本文采用Prandtl公式计算基坑坑底的抗隆起安全系数[1]，其表达式为：

Fs=γ2DNq+cNcγ1(H+D)+q	（2）

式中， Nq=eπ⋅tanφ⋅tan245°+ϕ2,Nc=Nq−1⋅cotϕ，c,ϕ为土体强度参数；γ1,γ2为基坑外、内的土体重度。依据公式（2）和表1中工程实例提供的参数，计算能够得到11个基坑工程的抗隆起安全系数（其它21个基坑数据不完备），绘制了基坑坑底抗隆起安全系数与最大地面沉降量关系曲线，如图4所示。

图5 基坑抗隆起安全系数与最大地面沉降关系

Fig .5  Relationship between safety coefficient against heave(Fs)        and normalized maximum ground settlement (δvm/H)

从图5可以看出，抗隆起安全系数Fs增大，最大地面沉降量减小。另外，本文提出的曲线和上海隧道院推荐的曲线进行了对比，基本保持一致，说明了两条曲线均可以用来预测基坑周围最大沉降量。

3  基坑周围地面沉降预测方法

Peck以1969年提出了采用正态密度函数预测隧道施工引起地面沉降的方法[2]，取得了较好的预测效果。图2的研究表明，上海市基坑周围地面沉降曲线型式以凹槽型为主，存在一个沉降槽偏心距x0。因此，笔者基于正态密度分布函数提出了考虑沉降槽偏心距x0的地面沉降预测公式：

δv=δvm×e−k(x−x0)2	（3）

式中，k为沉降相关系数，与沉降槽的影响半径相关。x0为沉降槽的偏心距（见图1），即最大沉降点所在的位置。当x0＞0时，沉降曲线表现为凹槽型；当x0=0时，沉降曲线表现为抛物型。

根据公式（3），如果知道了参数δvm、k和x0的大小，就可以求出任意一点的地面沉降量δv。δvm可以根据Fs的大小查阅图4得到，当Fs求解困难时，也可以根据以下步骤计算得到：

步骤1：对公式(3)两边同时取对数，整理得到：

lnδv=lnδvm−k(x−x0)2	（4）

令y=lnδv，ym=lnδvm，z=(x−x0)2，则公式（4）转换为：

y=ym−kz	（5）

把z看作自变量，则y是关于z的线性函数。

步骤2：根据监测成果，选取不同的xi(i≥3)值以及对应的δvi值，令x0=0,±0.1H,±0.2H,⋯,±1.0H，分别代入公式(4)、（5），可以得到一系列y、z值。因为y和z满足线性关系，对其进行线性回归拟合，可以得到ym、k值及判定系数R2。

步骤3：绘制x0与R2的关系图，确定x0的初值大小x0'。

步骤4：再令

x0=x0',x0'±0.01H,x0'±0.02H,⋯,x0'±0.09H，重复步骤2和步骤3，确定x0的最终值。

步骤五：把求得的x0值代入到步骤2，可以求得ym、k的最合适值，即可求得k和δvm的大小。

4基坑实例验证

以上海市金融中心基坑工程[13]为例，说明本文提出的预测方法的实施步骤。

⑴该基坑开挖深度16.9m，监测得到基坑周围六个点的沉降资料，监测点距基坑的距离 xi和沉降量δvi(i=1,2,…,6)，如表2所示。

表2 基坑沉降点资料

Tab. 2 Spots’ settlement data

| 数据 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| x/m | 7.0 | 12.2 | 17.2 | 19.3 | 25.0 | 30.0 |
| δv/mm | 60.8 | 72.4 | 65.4 | 56.0 | 41.0 | 31.4 |

⑵因为基坑开挖深度为16.9m，所以令x0=0,±1.7,±3.4,⋯,±17.0，代入公式（4）、（5），得到相应的y、z值，对y、z进行线性拟合，可以得到线性拟合方程。限于文章篇幅，图6仅给出了当x0=±10.2,±11.9时的拟合直线方程。

图6 线性拟合方程（x0=±10.2, ±11.9）

Fig. 6 Equation of the linearity fitting(x0=±10.2, ±11.9)

⑶根据不同的x0值及对应的R2值，绘制两者的相关关系图，如图7所示。从图7可以发现，x0=0.7H时，R2取最大值，即得到其初始值x0'=0.7H=11.9m。

图7 x0与R2关系图

Fig. 7 Relationship between x0 and R2

⑷令x0=11.9,11.9±0.17,11.9±0.34,⋯,11.9±1.53，同理，按照（2）和（3）的方法，可以得到更精确的x0值，最终结果为x0=11.90m（过程略）。

⑸把x0=11.90m代入公式(4)、（5），拟合得到其线性表达式，如图6所示。可以得到k=−0.0025，δvm=e4.208=67.2mm。把k、δvm代入公式（3），即得到预测该基坑地面沉降的表达式。

笔者应用该方法同时计算预测了其它三个基坑工程实例，相关参数的计算结果见表3。需要说明的是，对于新世纪商厦基坑工程，其偏心距x0小于0，理论上的δvm发生在基坑坑壁内侧，与实际显然不符，故应取其在坑壁处的沉降量作为最大沉降量。

表3 四个基坑实例相关参数的计算结果

Tab. 3 Result of relevant parameters four cases

| 基坑工程名称 | δvm/mm | k(10-3) | x0/m | R2 |
| --- | --- | --- | --- | --- |
| 上海市金融中心 | 67.2 | -2.5 | 11.90 | 0.9632 |
| 胜康廖氏大厦 | 130.3 | -5.4 | 19.32 | 0.9281 |
| 新世纪商厦 | 141.8 | -2.4 | -25.05 | 0.9865 |
| 延长路地铁车站 | 30.9 | -4.6 | 7.08 | 0.9352 |

图8给出了上述四个基坑的预测值和监测值的对比曲线，可以看出，无论是凹槽型还是抛物型的沉降曲线，曲线吻合效果都较好，说明预测精度较高。

| (a)上海市金融中心 | (b)胜康廖氏大厦 |
| --- | --- |
| (c)新世纪商厦 | (d)延长路地铁车站 |

图8 预测曲线与监测曲线对比

Fig. 8 Contrast of the predictive curves and monitoring curves

5  结论

本文通过对上海32个成功基坑工程实测资料的收集与分析，探讨了基坑周围的地面沉降特点，提出了考虑沉降槽偏心距的地面沉降预测方法，得到以下结论：

⑴基坑周围地面沉降曲线以凹槽型为主，存在沉降槽偏心距。

⑵最大沉降量δvm具有以下特点：δvm变化范围在0.4δhm～1.9δhm之间，其平均值和δhm相等；对于上海地区采用连续墙加内支撑作为围护结构的基坑，支撑系统相对刚度都比较大，对最大沉降量影响很小；抗隆起安全系数越大，基坑最大沉降量越小。

⑶基坑工程实例验证表明，本文提出的考虑沉降槽偏心距的地面沉降预测方法精度较高，操作方便，是一种实用的基坑周围地表沉降预测方法。

参考文献：

[1] 刘建航,侯学渊.基坑工程手册[M].北京: 中国建筑工业出版社,1997.

LIU Jianhang, HOU Xueyuan. Excavation engineering handbook[M]. Beijing: China Construction  Press, 1997.

[2] PECK R B. Deep excavations and tunneling in soft ground. State-of-the-art report[C]// Proc 7th Int Conf Soil Mechanics Foundation Engineering.Mexico:[s.n.], 1969: 225–290.

[3] CLOUGH R W, O’ROURKE T D. Construction induced movements of in-situ walls[C]// Proceedings, Design and Performance of Earth Retaining Structures, ASCE Special Conference. New York. :[s.n.], 1990: 439–470.

[4] OU C Y, HSIEH P G., CHIOU D C. Characteristics of ground surface settlement during excavation[J]. Canadian Geotechnical Journal, 1993, 30(5): 758.

[5] 刘建航.地下墙深基坑周围地层移动的预测和治理之二[J].地下工程与隧道,1993,(2):2.

LIU Jianhang. Prediction and government of strata movement around deep excavation supported by cast in Situ Wall[J]. Underground engineering and tunnels, 1993,(2):2.

[6] 唐孟雄,赵锡宏.深基坑周围地面沉降与变形分析[J].建筑科学,1996,4:31

TANG Mengxiong, ZHAO Xihong. Analysis of  ground settlement around deep excavation[J].Building Sciences. 1996,4:31.

[7] 黄强, 惠永宁. 深基坑支护工程实例集[M]. 北京: 中国建筑工业出版社, 1997.

HUANG Qiang, HUI Yongning. Deep excavation engineering case histories[M]. Beijing: China Architecture and Building Press, 1997.

[8] 王显星,包旭范,高强.上海铁路南站大型基坑工程施工对周边环境影响[J].岩石力学与工程学报,2005,24（增）:5454.

WANG Xianxing, BAO Xufan, GAO Qiang. Influence of large foundation pit construction caused by Shanghai south railway station on environment in Shanghai [J]. Chinese Journal of Rock Mechanics and Engineering. 2005,24（S）:5454.

[9] 李培国, 章凤仙. 人民广场地下变电站深基坑施工简介[J]. 岩土工程师, 1992, 4 (1): 41.

LI Peiguo, Zhang Fengxian. A brief introduction to the construction of the people's square substation [J]. Geotechnical Engineer, 1992, 4 (1): 41.

[10] 陆林强. 上海地铁上体馆主变电站工程施工与监测[J]. 隧道建设, 1993(3): 40.

LU Linqiang. Construction and monitoring of a deep excavation in subway station[J]. Tunnel Construction, 1993(3): 40.

[11] 夏明耀,曾进伦.地下工程设计施工手册[M].北京: 中国建筑工业出版社,1999.

XIA Mingyao, ZENG Jinlun. Underground engineering design handbook[M]. Beijing: China Construction  Press, 1999.

[12] 陈中汉,黄书秩,程丽萍.深基坑工程[M].北京: 机械出版社,2003.

CHEN Zhonghan, HUANG Shuzhi,CHENG Liping. Deep Excavation Engineering[M]. Beijing: Machine  Press, 2003.

[13] 王利民, 曾马荪. 深基坑工程周围建筑及围护结构的监测分析[J]. 建筑科学, 2000, 16(2): 35.

WANG Limin, ZENG Masun. Analysis of the monitoring data of the a deep excavation[J]. Building Science, 2000, 16(2): 35.

[14] 顾开云, 徐  磊. 复杂条件下基坑变形控制[J].岩土工程学报, 2006, 28(增): 1577.

GU Kaiyun, YU lei. Displacement control under deep foundation pits complicated conditions[J]. Chinese Journal of Geotechnical Engineering 2006, 28(S): 1577.

[15] 杨国祥, 李  侃, 赵锡宏, 等. 大型超深基坑工程信息化施工研究—上海外环隧道的浦西基坑工程[J]. 岩土工程学报, 2003, 25(4): 483.

YANG Gu-xiang, LI Kan, ZHAO Xi-hong, et al. A study on construction for specially big & deep excavation engineering—deep excavation engineering of outer ring tunnel project in Puxi, Shanghai[J]. Chinese Joumal of Geotechnical Engineering, 2003, 25(4): 483.

[16] G.B. Liu, Charles W.W.Ng, Z.W. Wang. Observed Performance of a Deep Multistrutted Excavation in Shanghai Soft Clays[J].Journal of Geotechnical and Geoenvironmental Enginnering,2005,131(8):1004.

[17] 王福恩.上海地铁M8线黄兴路车站深基坑综合施工技术研究[D]. 成都: 西南交通大学土木工程学院, 2003.

WANG Fuen. Research on the construction technology of M8 Line at Huangxing Station in Shanghai[D]. Chengdu: Southwest Jiaotong University. School of Civil Engineering, 2003.

[18] 黄海滨.上海地铁车站施工中的环境保护技术[D]. 上海:同济大学土木工程学院, 2000.

HUANG Hai-bin. The environmental protection during the construction of subway in Shanghai[D]. Shanghai: Tongji University. School of Civil Engineering，2007, 2000.

[19] 刘燕，刘涛，胡承军等.上海地铁M8线某车站基坑开挖变形特征研究[J].浙江工业大学学报,2005,33（5）：547.

LIU Yan, LIU Tao,HU Chengjun. Research of deformation laws of shanghai metro line & station pit [J]. JOURNAL OF ZHEJIANG UNIVERSITY OF TECHNOLOGY ,2005,33（5）:547.

[20] 徐方京. 上海地铁规划及其建设－盾构隧道施工和深开挖分析[J]. 岩土工程师, 1992, 4(2): 7.

XU Fangjing. Layout and construction of subways in Shanaghai—analysis of its construction and deep excavation[J]. Geotechnical Engineers, 1992, 4(2): 7.

[21] 侯学渊, 夏明耀, 李桂花. 软土深基坑工程的稳定与隆起研究[R]. 上海: 同济大学土木工程学院, 1991.

HOU Xueyan, XIA Mingyao, LI Guihua. Research on the stability and uplift of deep excavation in soft soil[R]. Shanghai: Tongji University. School of Civil Engineering, 1991.

[22] 赵锡宏, 陈志明. 高层建筑深基坑围护工程实践与分析[M]. 上海: 同济大学出版社, 1996.

ZHAO Xihong, CHEN Zhiming. The practice and analysis of deep foundation pit to high-rise building[M]. Shanghai Tongji University Press, 1996.

[23] 王元湘.深基坑挡土结构的受力分析[J].土木工程学报,1998,31(2):12.

WANG Yuanxiang. Analysis on force of retaining structure in deep excavation[J]. China Civil Engineering Journal, 1998, 31(2): 12.

[24] 刘金元. 软土基坑的离心模型试验研究[D]. 上海: 同济大学土木工程学院, 1999.

LIU Jin-yuan. Research on the centrifugal model test of deep excavations in soft soil[D]. Shanghai: Tongji University. School of Civil Engineering, 1999.

[25] 聂宗泉. 上海地铁M8线延吉中路站基坑变形控制技术[J].施工技术, 2005,34(10): 61.

NIE Zongquan. Control technology on deformation of deep foundation pit of Yanji Street Station of M8 Line of Shanghai subway[J]. Construction Technology, 2005, 34(10): 61.

[26] 王  琼. 上海浦西第一楼—恒隆广场大厦18.2 m基坑工程 的设计[J]. 建筑施工, 1999, 21(6): 1.

WANG Qiong. 18.2 m deep foundation pit engineering design of Shanghai Henglong Plaza [J]. Building Construction, 1999, 21(6): 1.

[27] 李永盛, 胡志明. 深幅地下连续墙基坑围护结构的变形与受力[J]. 建筑施工, 1995, 17(3): 19.

LI Yong-sheng, HU Zhi-ming. Deformation and mechanics of deep excavation supported by diaphragm wall[J]. Building Construction, 1995, 17(3): 19.

[28] ZHAO Xihong, GONG Jian, CHEN Zhiming. Design & pratice on engineering projections of specially deep & big excavation in Shanghai[J]. Chinese Journal of Geotechnical Engineering, 1999, 1(1): 104.

[29] 基坑工程实例编辑委员会. 基坑工程实例1[M]. 北京: 中国建筑工业出版社, 2006.

Commission of Deep Excavation Engineering Case History. Deep excavation engineering case history 1[M]. Beijing: China Architecture and Building Press, 2006.

[30] 刘丽萍,王胜利,章小民.无内衬地下连续墙的变形影响及控制[J].西安工业大学学报,2007,27(3):279.

LIU Li-ping,WANG Sheng-li,ZHANG Xiao-min. Influence of deformation of underground curtain without interior wall and its control measures[J]. 2007,27(3):279.

[31] OU C Y,LAI C.H. Finite Element Analysis of Deep Excavation in Layered Sandy and Clayey Soil Deposits[J]. Canadian Geotechnical Journal,1994,31(5):204.